# RCC-E ACI Design Discussion – 26 August 2025

## Summary Document

### Purpose

To discuss the redesign of the ACI (Application Centric Infrastructure) strategy for RCC-E, address network integration challenges, upgrade paths, IPv6 transition, firewall strategy, and next steps.

### Key Participants

- Eric Helgeson – Engineer, WWT Europe

- Rita Younger – Practice Lead, Data Center Networking, WWT

- John Barber – Network Consulting Engineer (NCE), Clay Concern

- Belete Ageze – Customer Delivery Architect, Cisco Public Sector

### Main Discussion Points

#### 1. Current State and Priorities

- Existing infrastructure includes legacy Cisco 5K switches and FEXes, currently being migrated into new ACI leaf switches (9Ks).

- Focus is on replacing legacy equipment before major ACI redesign.

- Ongoing use of Python/Ansible automation for configuration migration.

#### 2. ACI Redesign Approach

- Considering single-tenant model with several VRFs, bridge domains, and EPGs.

- Gradual move from network-centric to more application-centric segmentation as application dependencies are mapped.

- Initial recommendation to migrate all workloads to a single ESG, then segment as needed.

#### 3. Integration and Migration

- Immediate goal: VMM (Virtual Machine Manager) integration to streamline VMware operations.

- Challenges around application dependency mapping; critical for effective segmentation.

#### 4. Firewall and Security Strategy

- Current design hairpins inter-VRF traffic through firewalls for routing and security.

- Consensus: ESGs provide adequate segmentation; introducing additional east-west firewalls may add unnecessary complexity.

- No current need for complex traffic engineering within ACI.

#### 5. IPv6 Transition

- Top priority for the customer is dual-stack IPv4/IPv6 support.

- Current limitations: Underlay IPv6 not yet supported in ACI Multi-site; overlay IPv6 is supported for user/application traffic.

- Underlay IPv6 support anticipated in future ACI releases.

#### 6. Upgrade and Tooling

- Need to upgrade ACI and NDO/NDFC to access new features (e.g., full ESG support).

- Ongoing reliance on Cisco Nexus Dashboard and associated tools.

#### 7. Next Steps & Recommendations

- Complete replacement of 5K infrastructure.

- Accelerate VMM domain integration.

- Start with network-centric mode, transition to application-centric as dependencies are mapped.

- Collaborate on diagrams and documentation for better situational awareness.

- Plan workshops to accelerate design decisions and ensure alignment.

- Monitor ACI/NDO software roadmap for IPv6 and ESG enhancements.

## Detailed Document

### 1. Team Introductions and Roles

- Eric Helgeson: WWT Europe engineer, 10+ years supporting the region, provides ongoing ACI expertise.

- Rita Younger: 30+ years with Cisco networks, leads data center networking practice, experienced consultant and instructor.

- John Barber: Onsite NCE at Clay Concern, specializes in data center and ACI, responsible for on-the-ground migration and design.

- Belete Ageze: Cisco public sector architect, supports migration and ACI best practices.

### 2. ACI Redesign Goals and Strategy

- Drivers: Equipment refresh (migration from 5Ks to 9Ks), maximize ACI investment, and improve segmentation/security.

- Design Approach:

  - Single Tenant: Considered optimal given multi-site structure and potential future site consolidation.

  - Current Segmentation: 8–9 VRFs, ~192 EPGs/bridge domains.

  - Migration Path: Move existing infrastructure into ACI using network-centric mode; transition to ESGs and application-centric segmentation over time.

  - VMM Integration: Identified as a priority for operational efficiency.

### 3. Network Integration and Application Mapping

- Challenges:

  - Lack of application dependency mapping makes true application-centric design difficult.

  - Plan to begin with all workloads in a single ESG, gradually segment as critical applications are identified.

  - Use of ServiceNow for potential dependency mapping; Cisco tools considered cost-prohibitive.

- Tools: Python/Ansible automation for migration; NDI for limited dependency insights.

### 4. Firewall and Routing Design

- Current State: All inter-VRF routing is via external firewalls (“hairpinning”).

- Discussion: No current requirement for complex in-fabric firewalling; ESGs suffice for most segmentation and security needs.

- Future Considerations: Avoid overcomplicating network with unnecessary east-west firewalls.

### 5. IPv6 Transition and Dual Stack Support

- Customer Priority: Move to dual-stack IPv4/IPv6 throughout the data center.

- ACI Support:

  - Overlay IPv6: Supported for endpoints and bridge domains.

  - Underlay IPv6: Not yet supported in ACI multi-site; expected in future releases (potentially 6.2.x).

  - Workarounds: IPv4 underlay with IPv6 overlay is currently functional for most user/application traffic.

  - Limitations: VXLAN tunnels over ISN currently require IPv4; full native IPv6 underlay support pending.

### 6. Upgrade and Platform Considerations

- ACI/NDO/NDFC Upgrades: Required for latest ESG and IPv6 features. Current versions may limit some multi-site capabilities.

- Alternative Solutions: Some customers are evaluating NDFC over ACI for simplicity; Arista considered but found more complex.

### 7. Collaboration and Next Steps

- Action Items:

  - Share updated network diagrams for design alignment.

  - Schedule workshops (potentially involving broader teams) to finalize migration and design plans.

  - Monitor software release notes for critical features (IPv6 underlay, enhanced ESG).

  - Engage in peer reviews and external validation to support internal alignment.

- Workshop Structure: Led by on-site NCE, supported by WWT/Cisco experts, focus on actionable design decisions.

- Customer Environment: Typical of DoD and enterprise clients; balancing legacy constraints with modern ACI features.

### 8. Additional Observations

- Industry Trends: Most customers remain in network-centric mode due to lack of application mapping; complexity has led some to consider alternatives.

- Support Considerations: Cisco support and ecosystem remain a differentiator versus competitors like Arista.

## Attachments

(Please attach your diagrams or supporting documents as referenced.)

### Example: Action Item Table

| Action Item | Owner | Due Date | Status |
| --- | --- | --- | --- |
| Replace legacy 5Ks/FEXes | John Barber | Q4 2025 | In Progress |
| Integrate VMM domains | John Barber | Q4 2025 | Pending |
| Share latest network diagrams | John Barber | Q3 2025 | Pending |
| Plan and schedule workshop | Eric Helgeson | Q3 2025 | Pending |
| Monitor ACI upgrade roadmap | Rita Younger | Ongoing | In Progress |

### Notes & Considerations

- Edge Cases: If IPv6 underlay is required before support is available, project timelines may need adjustment.

- Dependencies: Migration sequencing is dependent on completion of hardware upgrades and VMM integration.

- Customer Prioritization: Shifts in DOD requirements or site consolidation may affect design direction.

# RCC-E ACI Design Discussion – Detailed Meeting Record

Date: 26 August 2025
Session: RCC-E ACI Design Discussion-20250826 1347-1
Generated: AI-assisted, based on provided transcript

## Table of Contents

- Participants & Roles

- Meeting Overview

- Current State of Environment

- Project Priorities & Drivers

- ACI Redesign Discussion

- Technical Migration Details

- Firewall & Security Architecture

- IPv6 Transition & Multi-Site Considerations

- ACI/Nexus Dashboard Tooling & Upgrade Path

- Challenges and Risks

- Next Steps & Action Items

- Appendices: Key Quotes and Insights

## 1. Participants & Roles

| Name | Role/Org | Responsibilities/Context |
| --- | --- | --- |
| Eric Helgeson | Engineer, WWT (Europe) | Regional ACI lead, customer advisor, Europe experience |
| Rita Younger | Practice Lead, DC Networking, WWT | 30+ yrs Cisco, design lead, consultant, instructor |
| John Barber | NCE, Clay Concern | Onsite engineer, ACI migration, config, project advocate |
| Belete Ageze | Customer Delivery Architect, Cisco | US Public Sector, Cisco ACI expert, design guidance |
| Mike Woody | (Not present, referenced) | ACI technical SME/lab developer, resource |

## 2. Meeting Overview

- Purpose: Align stakeholders on ACI redesign, network integration, migration priorities, IPv6 support, and next steps for RCC-E.

- Format: Open technical discussion with design brainstorming, operational review, and Q&A.

- Chapters: Introductions, ACI redesign, integration challenges, firewall/IPv6 strategy, upgrade path, networking challenges, wrap-up.

## 3. Current State of Environment

- Data Center Fabric:

  - Legacy: Cisco 5Ks, FEXes (acting as Layer 2 access below 9Ks).

  - Target: New ACI leaf switches (9Ks) to replace 5Ks/FEXes.

- ACI Deployment:

  - Tenancy: Single tenant, multi-site (two main, one remote leaf).

  - Segmentation: 8–9 VRFs, ~192 EPGs and bridge domains.

  - External Connections: L3Outs to firewalls per site; ISN (Inter-Site Network) used for site-to-site.

- Tools/Automation: Python and Ansible for extracting configs and deploying to ACI; intention to leverage automation for port bindings, minimize VLAN sprawl.

## 4. Project Priorities & Drivers

- Immediate: Remove 5Ks/FEXes, migrate workloads to ACI 9Ks.

- Mid-term: Redesign ACI for greater efficiency, security, and application alignment.

- Other: Transition to dual-stack IPv4/IPv6, improve VMware (VMM) integration, simplify segmentation/security, maximize investment in ACI.

## 5. ACI Redesign Discussion

### 5.1. Design Philosophy

- Current Mode: Network-centric, mapping VLANs/EPGs/VRFs directly.

- Target Mode: Gradual shift to application-centric segmentation (using ESGs), as application dependencies are mapped.

- Single vs. Multi-Tenant: Consensus to remain single-tenant for current and near-future, pending site consolidation.

### 5.2. Segmentation Approach

- Initial Step: Migrate all workloads into one ESG for simplicity and control; break out into multiple ESGs/contracts as more is learned about application dependencies.

- Mapping: VRFs aligned with bridge domains/EPGs; VLANs mapped to ESGs.

- Application Discovery: No comprehensive mapping currently; plan to use ServiceNow and limited NDI capabilities.

### 5.3. Application Dependency Mapping

- Challenge: No current deep visibility into application flows; risk of “breaking” critical apps during segmentation.

- Best Practice: Start with low-risk apps for segmentation experiments; maintain broad connectivity until dependencies are mapped.

### 5.4. VMM Integration

- Critical Priority: VMM domains (for VMware) not yet integrated; viewed as a first technical milestone for improved operational efficiency.

## 6. Technical Migration Details

- Current Tasks:

  - Extract configs from 5Ks using Ansible/Python.

  - Rebind ports and migrate VLANs to ACI fabric.

- Bindings: Focus on proper EPG/port bindings, avoiding unnecessary VLAN carryover.

- Legacy Complexity: 5Ks currently act as access, pass all VLANs through, many endpoints (especially VMware and UCS) hang off FEXes.

- Clean-Up Goal: Migrate all access to 9Ks, simplify fabric, and remove “ugly” legacy L2 topology.

## 7. Firewall & Security Architecture

### 7.1. Current Model

- Inter-VRF Routing: All routed via external firewall (hairpin).

- East-West Security: ESGs used for segmentation; no in-fabric firewalls.

### 7.2. Design Discussion

- Consensus: ESGs sufficient for most segmentation/security; avoid adding in-fabric firewalls unless specific compliance/engineering needs arise.

- Complexity Warning: Inserting firewalls for east-west traffic in ACI increases complexity, especially in a multi-site context; consensus to avoid unless strictly required.

- Efficiency: Some minor concerns about hairpin routing efficiency, but not identified as a bottleneck.

## 8. IPv6 Transition & Multi-Site Considerations

### 8.1. Customer Priority

- Dual-Stack: Strong DOD-driven push to enable IPv6 everywhere; dual-stack (not IPv6-only) is the immediate goal.

- Bridge Domains/L3Outs: Need to support IPv6 addressing and routing throughout.

### 8.2. ACI Support

- Overlay IPv6: Supported—endpoints and bridge domains can use IPv6.

- Underlay IPv6: Not currently supported in ACI Multi-site; VXLAN tunnels over ISN must use IPv4 underlay. IPv6 underlay targeted for future (possibly version 6.2+).

- Routing: BGP underlay remains IPv4; overlay (user/app traffic) can be dual-stack.

### 8.3. Implications

- User Traffic: Can achieve dual-stack endpoint-to-endpoint communication, even across sites, as long as underlay stays IPv4.

- Limitations: No native IPv6-only underlay or VXLAN tunnel establishment; may not matter for current workloads/applications.

## 9. ACI/Nexus Dashboard Tooling & Upgrade Path

- Current Tools: NDI (Nexus Dashboard Insights), NDO (Orchestrator), limited NDFC (Fabric Controller).

- Upgrade Need: ESG multi-site, enhanced IPv6 features require upgrade (current ACI ~6.0.7, NDO ~4.1).

## 10. Challenges and Risks

- Application Dependency Mapping: Lack thereof increases risk in segmentation efforts.

- Legacy Infrastructure: Prolonged use of 5Ks/FEXes complicates migration and increases operational risk.

- Customer Readiness: Internal delays, shifting priorities (e.g., IPv6, firewall refresh) could impact timelines.

- Tooling Gaps: Full benefit of Cisco’s advanced mapping/discovery tools not available due to cost constraints.

## 11. Next Steps & Action Items

| Action/Decision | Owner(s) | Target Date | Notes |
| --- | --- | --- | --- |
| Complete 5K/FEX migration to 9Ks | John Barber | Q4 2025 | Use automation for config/port migration |
| Integrate VMM domains for VMware | John Barber | Q4 2025 | First technical milestone |
| Share updated network diagrams | John Barber | Q3 2025 | For design review by Rita/Eric |
| Plan/schedule ACI redesign workshop | Eric Helgeson, Rita | Q3–Q4 2025 | Include all relevant stakeholders |
| Map initial application dependencies | John Barber | Q4 2025 | Use ServiceNow, NDI, interviews |
| Monitor ACI/NDO software roadmap | Rita Younger, Belete | Ongoing | IPv6 underlay, ESG improvements |
| Prepare documentation for firewall/F5 flows | John Barber | Q4 2025 | For future migration design |

## 12. Appendices: Key Quotes and Insights

- “The biggest struggle really is mapping everything, right? All the applications… it's not even smart, right? You can't maintain that.”

- “Right now, all inter-VRF routing goes through the firewall—no route leaking in ACI. Avoiding that keeps things cleaner.”

- “VMN integration should be first. That will make things so much easier from a virtual VMware standpoint.”

- “IPv6 overlay is supported, but underlay is not yet—shouldn’t be a blocker for most user traffic. Underlay support is on the roadmap.”

- “Many customers stay network-centric due to lack of application mapping. Even Cisco has moved away from pure ‘application-centric’ language.”

- “If hairpinning traffic through the firewall isn’t causing issues, don’t add complexity.”

- “NDFC is being adopted by some for simplicity, but ACI remains common for DOD/Enterprise due to support.”

## Notes and Special Considerations

- Edge Cases: If IPv6 underlay becomes urgent, project may require architecture shift or timeline adjustment.

- Dependencies: Complete removal of 5Ks is prerequisite for major ACI design changes.

- External Input: Workshop should include stakeholders managing F5, firewalls, and other critical integrations for holistic design.