# IPv6 Network Deployment for Cisco ACI Multi-Site

## Regional Cyber Center Europe (RCC-E) Infrastructure

## Overview

This project automates the deployment of IPv6 network infrastructure for Regional Cyber Center Europe (RCC-E) on Cisco ACI Multi-Site using Terraform and Python. It creates a complete IPv6 network topology with 33 Bridge Domains (BDs), 33 Endpoint Groups (EPGs), and automatic port binding configuration across two data center sites in Germany.

## Project Purpose

Deploys a standardized IPv6 addressing scheme for RCC-E services with:

- Function-code based IPv6 addressing: Each service gets a unique IPv6 subnet based on its function code

- Multi-site deployment: Resources stretched or localized based on service requirements across Grafenwöhr and Kaiserslautern

- Automated port bindings: Python script intelligently maps port bindings from existing IPv4 infrastructure

## Architecture

### Network Topology

┌─────────────────────────────────────────────────────────────┐

│ VRF-RCC-E (IPv6)                                            │

│ ┌───────────────────────────────────────────────────────┐   │

│ │ 33 Bridge Domains with IPv6 Subnets                   │   │

│ │ ┌─────────────────────────────────────────────────┐   │   │

│ │ │ 33 Endpoint Groups (EPGs)                       │   │   │

│ │ │ ┌───────────────────────────────────────────┐   │   │   │

│ │ │ │ L2_Stretched (29 EPGs)                    │   │   │   │

│ │ │ │ - AEDCG Site (Grafenwöhr)                 │   │   │   │

│ │ │ │ - AEDCK Site (Kaiserslautern)             │   │   │   │

│ │ │ └───────────────────────────────────────────┘   │   │   │

│ │ │ ┌───────────────────────────────────────────┐   │   │   │

│ │ │ │ L2_Non-Stretched (2 EPGs)                 │   │   │   │

│ │ │ │ - DB-SVR (site-local)                     │   │   │   │

│ │ │ │ - SYSLOG (site-local)                     │   │   │   │

│ │ │ └───────────────────────────────────────────┘   │   │   │

│ │ │ ┌───────────────────────────────────────────┐   │   │   │

│ │ │ │ G-Specific_Only (1 EPG)                   │   │   │   │

│ │ │ │ - GEF-MGMT (Grafenwöhr only)              │   │   │   │

│ │ │ └───────────────────────────────────────────┘   │   │   │

│ │ │ ┌───────────────────────────────────────────┐   │   │   │

│ │ │ │ K-Specific_Only (1 EPG)                   │   │   │   │

│ │ │ │ - BACKUP-SVR (Kaiserslautern only)        │   │   │   │

│ │ │ └───────────────────────────────────────────┘   │   │   │

│ │ └─────────────────────────────────────────────────┘   │   │

│ └───────────────────────────────────────────────────────┘   │

└─────────────────────────────────────────────────────────────┘

### Data Center Locations

- AEDCG: Grafenwöhr, Germany

- AEDCK: Kaiserslautern, Germany

### IPv6 Addressing Scheme

All IPv6 addresses follow the pattern: [FUNCTION_CODE]00::1/56

Examples:

- Function 15 (NAC) → 1500::1/56 → VLAN 3021

- Function 69 (CFG-MGMT) → 6900::1/56 → VLAN 3105

- Function db (DB-SVR) → db00::1/56 → VLAN 3219

## Components

### 1. Terraform Configuration (bds_epgs.tf)

Purpose: Defines and deploys the complete IPv6 network infrastructure

What it creates:

- 1 VRF (Virtual Routing and Forwarding): VRF-RCC-E

- 1 Contract: Any_VRF-RCC-E for inter-EPG communication

- 4 Application Profiles across different templates

- 33 Bridge Domains with IPv6 subnets

- 33 Endpoint Groups

- 66 Site-specific EPG associations

- 63 Site BD configurations

- 66 Physical domain associations

Resource Distribution:

- L2_Stretched Template: 29 BDs/EPGs (active-active across both sites)

- L2_Non-Stretched Template: 2 BDs/EPGs (site-local instances)

- G-Specific_Only Template: 1 BD/EPG (Grafenwöhr site only)

- K-Specific_Only Template: 1 BD/EPG (Kaiserslautern site only)

### 2. Python Binding Script (ipv6_binding_generator.py)

Purpose: Automatically configures static port bindings for IPv6 EPGs

What it does:

- Discovers all IPv6 RCC-E EPGs from the deployed schema

- Extracts port bindings from existing IPv4 EPGs (reference architecture)

- Maps IPv6 EPGs to appropriate IPv4 reference EPGs based on function

- Assigns correct VLANs based on function codes

- Filters port bindings by template type and site

- Deploys static port configurations to NDO

- Excludes leaves 101/102 from all bindings

Key Features:

- Intelligent mapping based on service function

- VLAN conflict detection

- Template-aware filtering (respects G-Specific, K-Specific, Stretched, Non-Stretched)

- Batch deployment with progress tracking

- Comprehensive verification reporting

## Prerequisites

### Required Software

- Terraform >= 1.0

- Python >= 3.7

- Cisco MSO/NDO Provider for Terraform

- Python packages: requests, urllib3

### Network Requirements

- Access to Cisco NDO API (198.18.1.12)

- Schema "AEDCE" must exist

- Sites "AEDCG" (Grafenwöhr) and "AEDCK" (Kaiserslautern) must be configured

- Tenant "EUR" must exist

- Physical domain "PhysDom_ACI_Nexus" must exist

- Templates must exist:

  - UpgradeTemplate1

  - L2_Stretched

  - L2_Non-Stretched

  - G-Specific_Only

  - K-Specific_Only

### Python Virtual Environment

IMPORTANT: The Python script requires a virtual environment located at:

/Users/johbarbe/

The virtual environment MUST be activated before running the Python script.

See SETUP.md for detailed virtual environment setup instructions.

## Quick Start

### 1. Prepare Terraform

git clone <repository-url>

cd <repository-directory>

terraform init

terraform plan

### 2. Deploy Infrastructure

terraform apply

Type 'yes' when prompted.

Expected deployment time: 5-10 minutes

### 3. Configure Port Bindings

source /Users/johbarbe/bin/activate

python3 ipv6_binding_generator.py both

Type 'yes' when prompted to deploy.

Expected binding deployment time: 2-5 minutes

## Function Code Reference

### Complete Mapping Table

| Function | Purpose | IPv6 Subnet | Gateway | VLAN | Public | Template |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | NAC | 1500::/56 | 1500::1/56 | 3021 | No | L2_Stretched |
| 1b | LB | 1b00::/56 | 1b00::1/56 | - | No | L2_Stretched |
| 40 | VVOIP-MGMT | 4000::/56 | 4000::1/56 | 3064 | No | L2_Stretched |
| 41 | VVOIP-PROXY | 4100::/56 | 4100::1/56 | 3065 | No | L2_Stretched |
| 53 | DNS-MGMT | 5300::/56 | 5300::1/56 | 3083 | No | L2_Stretched |
| 69 | CFG-MGMT | 6900::/56 | 6900::1/56 | 3105 | No | L2_Stretched |
| ad | AD | ad00::/56 | ad00::1/56 | 3173 | No | L2_Stretched |
| af | ADFS | af00::/56 | af00::1/56 | 3175 | No | L2_Stretched |
| bc | RCC-E-SVR | bc00::/56 | bc00::1/56 | - | No | L2_Stretched |
| bd | RCC-E-DNS | bd00::/56 | bd00::1/56 | - | No | L2_Stretched |
| be | RCC-E-DCO | be00::/56 | be00::1/56 | - | No | L2_Stretched |
| bf | RCC-E-UNIX | bf00::/56 | bf00::1/56 | - | No | L2_Stretched |
| c0 | ACAS-SCANNERS | c000::/56 | c000::1/56 | 3192 | No | L2_Stretched |
| c1 | C2C-SCANNERS | c001::/56 | c001::1/56 | 3442 | No | L2_Stretched |
| c5 | OCSP | c500::/56 | c500::1/56 | 3197 | No | L2_Stretched |
| ca | PKI-SRV | ca00::/56 | ca00::1/56 | - | No | L2_Stretched |
| cb | LMR | cb00::/56 | cb00::1/56 | - | No | L2_Stretched |
| d0 | PRINT-SVR | d000::/56 | d000::1/56 | 3208 | No | L2_Stretched |
| d1 | FILE-SVR | d100::/56 | d100::1/56 | 3209 | No | L2_Stretched |
| d2 | DHCP-SVR | d200::/56 | d200::1/56 | 3210 | No | L2_Stretched |
| d5 | SMTP-SVR | d500::/56 | d500::1/56 | 3213 | No | L2_Stretched |
| d6 | D64-PROXY | d600::/56 | d600::1/56 | - | No | L2_Stretched |
| d7 | RWEB-PROXY | d700::/56 | d700::1/56 | - | Yes | L2_Stretched |
| d8 | FWEB-PROXY | d800::/56 | d800::1/56 | - | Yes | L2_Stretched |
| d9 | SYSLOG | d900::/56 | d900::1/56 | 3217 | No | L2_Non-Stretched |
| db | DB-SVR | db00::/56 | db00::1/56 | 3219 | No | L2_Non-Stretched |
| dd | BACKUP-SVR | dd00::/56 | dd00::1/56 | 3221 | No | K-Specific_Only |
| e0 | APP-SVR | e000::/56 | e000::1/56 | 3224 | No | L2_Stretched |
| e3 | FMWR-SVR | e300::/56 | e300::1/56 | - | No | L2_Stretched |
| e4 | WEB-SVR | e400::/56 | e400::1/56 | 3228 | Yes | L2_Stretched |
| e9 | E911-SVR | e900::/56 | e900::1/56 | - | No | L2_Stretched |
| ec | MECM | ec00::/56 | ec00::1/56 | 3236 | No | L2_Stretched |
| ef | GEF-MGMT | ef00::/56 | ef00::1/56 | - | No | G-Specific_Only |

### Service Categories

| Category | Range | Services |
| --- | --- | --- |
| Infrastructure | 10 | NAC, CFG-MGMT, MECM, GEF-MGMT |
| Network Services | 20 | LB, DNS-MGMT, RCC-E-DNS, DHCP-SVR, SMTP-SVR |
| Voice & Comms | 30 | VVOIP-MGMT, VVOIP-PROXY, LMR, E911-SVR |
| Security | 40 | ACAS-SCANNERS, C2C-SCANNERS, OCSP, PKI-SRV |
| Directory | 50 | AD, ADFS |
| Proxy | 60 | D64-PROXY, RWEB-PROXY, FWEB-PROXY |
| Applications | 70 | APP-SVR, WEB-SVR, FMWR-SVR |
| RCC-E Services | 80 | RCC-E-SVR, RCC-E-DCO, RCC-E-UNIX |
| Storage | 90 | PRINT-SVR, FILE-SVR, BACKUP-SVR |
| Database/Logging | A0 | DB-SVR, SYSLOG |

## Deployment Workflow

### Phase 1: Infrastructure (Terraform)

- terraform init - Initialize Terraform

- terraform plan - Review what will be created

- terraform apply - Deploy infrastructure

- Result: VRF, BDs, EPGs created

- Duration: 5-10 minutes

### Phase 2: Port Bindings (Python)

- Activate virtual environment

- Run Python script

- Script discovers EPGs

- Script generates bindings

- Script deploys to NDO

- Duration: 2-5 minutes

## Key Features

- Function-code based IPv6 addressing

- Multi-site deployment (Grafenwöhr and Kaiserslautern)

- Automated port binding from IPv4 reference

- VLAN conflict detection

- Template-based resource organization

- Comprehensive error handling

## Troubleshooting

### Terraform State Errors

If you see "Unable to find EPG" errors:

rm terraform.tfstate*

terraform init

terraform apply

### Python Authentication Errors

Check credentials in script and verify NDO access:

ping 198.18.1.12

### No EPGs Found

Run Terraform first:

terraform apply

python3 ipv6_binding_generator.py both

## Support Documentation

- SETUP.md - Environment setup and prerequisites

- TECHNICAL_GUIDE.md - Deep technical details

- PRESENTATION_TALKING_POINTS.md - Executive summary

- QUICK_REFERENCE.md - One-page cheat sheet

- Inline comments in both files for detailed explanations

## Version History

### Current Version: 1.0.0

Initial Release:

- Complete IPv6 network infrastructure for RCC-E

- 33 BDs and EPGs across 4 templates

- Automated port binding generation

- Multi-site support (Grafenwöhr, Kaiserslautern)

- Function code-based addressing

Last Updated: November 17, 2025

Region Cyber Center Europe (RCC-E)