# Technical Deep-Dive Guide

## Regional Cyber Center Europe (RCC-E) IPv6 Infrastructure

Comprehensive technical documentation for IPv6 RCC-E network deployment.

## Table of Contents

- Architecture Overview

- Terraform Configuration Explained

- Python Script Technical Details

- IPv6 Addressing Design

- Deployment Mechanics

- API Interactions

## Architecture Overview

### High-Level Design

This solution deploys a complete IPv6 network infrastructure for Regional Cyber Center Europe (RCC-E) using Infrastructure as Code (IaC) principles:

┌──────────────────────────────────────────────────────────────┐

│ Cisco NDO/MSO                                                │

│ ┌────────────────────────────────────────────────────────┐   │

│ │ Schema: AEDCE                                          │   │

│ │ ┌──────────────────────────────────────────────────┐   │   │

│ │ │ Tenant: EUR                                      │   │   │

│ │ │ ┌────────────────────────────────────────────┐   │   │   │

│ │ │ │ VRF: VRF-RCC-E                             │   │   │   │

│ │ │ │ ┌──────────────────────────────────────┐   │   │   │   │

│ │ │ │ │ 33 Bridge Domains (BDs)              │   │   │   │   │

│ │ │ │ │ - IPv6 Subnets                       │   │   │   │   │

│ │ │ │ │ - Function code based                │   │   │   │   │

│ │ │ │ └──────────────────────────────────────┘   │   │   │   │

│ │ │ │ ┌──────────────────────────────────────┐   │   │   │   │

│ │ │ │ │ 33 Endpoint Groups (EPGs)            │   │   │   │   │

│ │ │ │ │ - Static port bindings               │   │   │   │   │

│ │ │ │ │ - Physical domain associations       │   │   │   │   │

│ │ │ │ └──────────────────────────────────────┘   │   │   │   │

│ │ │ └────────────────────────────────────────────┘   │   │   │

│ │ └──────────────────────────────────────────────────┘   │   │

│ └────────────────────────────────────────────────────────┘   │

│                                                              │

│ Sites: AEDCG (Grafenwöhr) ←→ AEDCK (Kaiserslautern)        │

└──────────────────────────────────────────────────────────────┘

### Template Strategy

The configuration uses 4 separate templates to optimize resource management for RCC-E:

#### 1. UpgradeTemplate1 (VRF Container)

- Purpose: Holds the VRF definition only

- Resources: VRF-RCC-E, contracts

- Reason: VRF shared across all other templates

#### 2. L2_Stretched (29 BDs/EPGs)

- Purpose: Services requiring cross-site redundancy

- Deployment: Active-active across Grafenwöhr and Kaiserslautern

- Characteristics:

  - layer2_stretch = true

  - intersite_bum_traffic = true

  - optimize_wan_bandwidth = true

#### 3. L2_Non-Stretched (2 BDs/EPGs)

- Purpose: Services requiring site locality

- Deployment: Independent instance per site

- Services: DB-SVR, SYSLOG

- Characteristics:

  - layer2_stretch = true (for future flexibility)

  - intersite_bum_traffic = false

  - ep_move_detection_mode = "none"

#### 4. G-Specific_Only (1 BD/EPG)

- Purpose: Grafenwöhr-exclusive services

- Deployment: AEDCG site only

- Service: GEF-MGMT

#### 5. K-Specific_Only (1 BD/EPG)

- Purpose: Kaiserslautern-exclusive services

- Deployment: AEDCK site only

- Service: BACKUP-SVR

## Terraform Configuration Explained

### File Structure: bds_epgs.tf

The Terraform file is organized in logical sections:

bds_epgs.tf (2000+ lines)

├── Header Comments (Documentation)

├── Data Sources (4 resources)

│   ├── data.mso_schema.existing (Schema AEDCE)

│   ├── data.mso_site.aedcg (Grafenwöhr)

│   ├── data.mso_site.aedck (Kaiserslautern)

│   └── data.mso_tenant.eur (Tenant EUR)

├── VRF Definition (1 resource)

│   └── mso_schema_template_vrf.vrf_rcc_e

├── Contract Definition (1 resource)

│   └── mso_schema_template_contract.contract_vrf_rcc_e

├── L2_Stretched Template (29 BDs + 29 EPGs)

│   ├── Application Profile: AppProf-RCC-E

│   ├── Infrastructure Management (3 BDs/EPGs)

│   ├── Network Services (5 BDs/EPGs)

│   ├── Voice & Communications (4 BDs/EPGs)

│   ├── Security Services (4 BDs/EPGs)

│   ├── Directory & Auth (2 BDs/EPGs)

│   ├── Proxy Services (3 BDs/EPGs)

│   ├── Application Servers (3 BDs/EPGs)

│   ├── RCC-E Services (3 BDs/EPGs)

│   └── Storage Services (2 BDs/EPGs)

├── Site Associations - L2_Stretched (58 resources)

│   ├── Site ANPs (2 resources)

│   ├── Site EPGs - Grafenwöhr (29 resources)

│   └── Site EPGs - Kaiserslautern (29 resources)

├── G-Specific_Only Template (1 BD + 1 EPG)

├── K-Specific_Only Template (1 BD + 1 EPG)

├── L2_Non-Stretched Template (2 BDs + 2 EPGs)

├── Site BD Configurations (63 resources)

└── Physical Domain Associations (66 resources)

### Key Terraform Resources

#### VRF Configuration

resource "mso_schema_template_vrf" "vrf_rcc_e" {

schema_id        = data.mso_schema.existing.id

template         = "UpgradeTemplate1"

name             = "VRF-RCC-E"

display_name     = "VRF-RCC-E"

layer3_multicast = false

vzany            = true

}

Key Parameters:

- vzany = true: Enables VRF-wide contract (allows inter-EPG communication)

- layer3_multicast = false: IPv6 doesn't use multicast in this design

Purpose: Creates the routing container for all RCC-E services

#### Bridge Domain Pattern

Every BD follows this pattern:

resource "mso_schema_template_bd" "bd_example" {

schema_id               = data.mso_schema.existing.id

template_name           = "L2_Stretched"

name                    = "BD-EXAMPLE"

display_name            = "BD-EXAMPLE"

description             = "Service Name - Function: XX - VLAN: XXXX"

vrf_name                = mso_schema_template_vrf.vrf_rcc_e.name

vrf_schema_id           = data.mso_schema.existing.id

vrf_template_name       = "UpgradeTemplate1"

layer2_unknown_unicast  = "proxy"

layer2_stretch          = true

unicast_routing         = true

intersite_bum_traffic   = true

optimize_wan_bandwidth  = true

}

Critical Parameters Explained:

- layer2_unknown_unicast = "proxy": ACI acts as ARP/ND proxy (required for IPv6)

- unicast_routing = true: BD acts as L3 gateway (anycast gateway model)

- layer2_stretch = true: Enables L2 extension between Grafenwöhr and Kaiserslautern

- intersite_bum_traffic = true: Allows broadcast/unknown/multicast between sites

- optimize_wan_bandwidth = true: Compresses inter-site traffic

#### Subnet Configuration

resource "mso_schema_template_bd_subnet" "bd_example_subnet" {

schema_id     = data.mso_schema.existing.id

template_name = "L2_Stretched"

bd_name       = mso_schema_template_bd.bd_example.name

ip            = "1500::1/56"

scope         = "public"

shared        = false

}

IPv6 Subnet Pattern:

- Format: [FUNCTION_CODE]00::1/56

- Gateway is always first IP: ::1

- /56 prefix allows 256 /64 subnets per function

#### EPG Configuration

resource "mso_schema_template_anp_epg" "epg_example" {

schema_id     = data.mso_schema.existing.id

template_name = "L2_Stretched"

anp_name      = mso_schema_template_anp.appprof_rcc_e_stretched.name

name          = "EPG-EXAMPLE"

display_name  = "EPG-EXAMPLE"

bd_name       = mso_schema_template_bd.bd_example.name

}

EPG Naming Convention:

- Pattern: EPG-[SERVICE-NAME]

- Must match corresponding BD: BD-[SERVICE-NAME]

#### Site Association

resource "mso_schema_site_anp_epg" "site_epg_example_g" {

schema_id     = data.mso_schema.existing.id

template_name = "L2_Stretched"

site_id       = data.mso_site.aedcg.id

anp_name      = mso_schema_template_anp.appprof_rcc_e_stretched.name

epg_name      = mso_schema_template_anp_epg.epg_example.name

}

Purpose: Associates template EPG with physical site for deployment (Grafenwöhr or Kaiserslautern)

#### Site BD Configuration

resource "mso_schema_site_bd" "bd_example_g" {

schema_id     = data.mso_schema.existing.id

bd_name       = mso_schema_template_bd.bd_example.name

template_name = "L2_Stretched"

site_id       = data.mso_site.aedcg.id

host_route    = true

}

host_route = true: Critical for proper IPv6 routing - advertises /128 host routes for endpoint mobility

#### Physical Domain Association

resource "mso_schema_site_anp_epg_domain" "epg_example_domain_g" {

schema_id            = data.mso_schema.existing.id

template_name        = "L2_Stretched"

site_id              = data.mso_site.aedcg.id

anp_name             = mso_schema_template_anp.appprof_rcc_e_stretched.name

epg_name             = mso_schema_template_anp_epg.epg_example.name

domain_type          = "physicalDomain"

domain_name          = "PhysDom_ACI_Nexus"

deploy_immediacy     = "immediate"

resolution_immediacy = "immediate"

depends_on           = [mso_schema_site_anp_epg.site_epg_example_g]

}

Purpose: Enables static port binding capability for physical server connectivity

## Python Script Technical Details

### Script Architecture: ipv6_binding_generator.py

Class: NDOIPv6BindingGenerator

├── __init__()                    # Initialize with NDO connection

├── _authenticate()               # Get API token

├── get_schema_id()              # Lookup schema AEDCE

├── discover_rcc_e_epgs()        # Find all IPv6 RCC-E EPGs

├── extract_all_ipv4_bindings()  # Get reference bindings

├── generate_ipv6_bindings()     # Create binding config

├── save_bindings_to_file()      # Export JSON

├── deploy_bindings()            # Push to NDO

├── _build_epg_cache()           # Index EPG locations

├── _get_default_bindings()      # Fallback bindings

└── _find_next_available_vlan()  # VLAN conflict resolution

### Intelligent EPG Mapping Algorithm

The script uses functional similarity to map IPv6 RCC-E EPGs to IPv4 reference EPGs:

self.epg_mapping = {

'EPG-NAC': {

'reference': 'EPG-V0015',    # Maps to IPv4 NAC EPG

'vlan': 3021,                # Verified from deployment data

'template': 'L2_Stretched',

'function': '15',

'subnet': '1500::/56'

},

}

Mapping Logic:

- Service Function Match: NAC → NAC, DNS → DNS, etc.

- Port Pattern Reuse: Copies port topology from IPv4

- VLAN Assignment: Uses verified VLANs from deployment data

- Template Filtering: Respects G-Specific, K-Specific, Stretched, Non-Stretched

### Port Binding Extraction

def extract_all_ipv4_bindings(self, schema):

for site in sites:

for anp in site['anps']:

for epg in anp['epgs']:

for port in epg['staticPorts']:

path = port['path']

# Skip management leaves 101/102

if re.search(r'/(?:paths|protpaths)-10[12](?:/|-10[12])', path):

continue

# Store binding

all_bindings[epg_name].append(binding)

Regex Explanation:

- paths-101 or paths-102: Individual leaf ports

- protpaths-101-102: VPC between leaves 101 and 102

- These are excluded because they're management infrastructure

### VLAN Assignment Logic

# Priority order for VLAN assignment:

# 1. Use verified VLAN from deployment data (highest priority)

# 2. Use function table VLAN (medium priority)

# 3. Auto-assign from safe range 3050-3062 (fallback)

verified_vlans = [3021, 3064, 3065, 3083, 3105, 3173, 3175, 3192,

3197, 3208, 3209, 3210, 3213, 3217, 3219, 3221,

3224, 3228, 3236, 3442]

if epg_name in self.epg_mapping:

vlan = mapping['vlan']  # Use verified VLAN

if vlan in vlan_assignments:

# Conflict detected! Find next available

vlan = self._find_next_available_vlan(vlan_assignments, vlan + 1)

Conflict Resolution: If VLAN already assigned, script automatically finds next available VLAN and logs the change.

### Deployment Batch Processing

batch_size = 50  # Deploy 50 bindings at a time

for i in range(0, len(all_patches), batch_size):

batch = all_patches[i:i + batch_size]

# PATCH request to NDO

response = self.session.patch(schema_url, json=batch)

# Rate limiting to avoid API throttling

time.sleep(0.5)

Why batching?

- NDO API has payload size limits (~50 operations)

- Prevents timeout on large deployments

- Allows progress tracking

- Enables partial success (some batches may fail without breaking entire deployment)

## IPv6 Addressing Design

### Function Code System

Each RCC-E service gets a unique function code (2-digit hex) that determines its IPv6 addressing:

Function Code → IPv6 Subnet → Gateway IP

15       →  1500::/56  →  1500::1/56

69       →  6900::/56  →  6900::1/56

db       →  db00::/56  →  db00::1/56

### Addressing Hierarchy

VRF-RCC-E

└── Bridge Domain (Function XX)

├── IPv6 Subnet: XX00::/56

│   ├── Gateway: XX00::1

│   ├── Usable: XX00::2 through XX00:ff:ffff:ffff:ffff:ffff

│   └── Endpoints: Auto-assigned via SLAAC or DHCPv6

└── Endpoint Group

└── Static Port Bindings (VLAN)

### Subnet Calculation

For function code 15 (NAC):

- Network: 1500::/56

- Gateway: 1500::1/56

- Usable Range: 1500::2 to 15ff:ffff:ffff:ffff:ffff:ffff:ffff:ffff

- Number of addresses: 2^72 = 4,722,366,482,869,645,213,696 addresses

### VLAN to Subnet Mapping

VLANs are assigned based on actual VM deployments in RCC-E:

VLAN 3021 → Function 15 → Subnet 1500::/56 (NAC)

VLAN 3064 → Function 40 → Subnet 4000::/56 (VVOIP-MGMT)

VLAN 3105 → Function 69 → Subnet 6900::/56 (CFG-MGMT)

VLAN 3173 → Function ad → Subnet ad00::/56 (AD)

Unspecified VLANs: Functions without VLANs in deployment data use safe range (3050-3062) to avoid conflicts.

## Deployment Mechanics

### Terraform Resource Dependencies

Terraform automatically handles dependencies:

VRF-RCC-E (created first)

↓

Contract (references VRF)

↓

Application Profiles (references schema)

↓

Bridge Domains (reference VRF)

↓

BD Subnets (reference BDs)

↓

EPGs (reference BDs)

↓

Site ANPs (reference templates)

↓

Site EPGs (reference template EPGs)

↓

Site BDs (reference template BDs)

↓

Physical Domains (reference site EPGs)

Terraform automatically orders creation - you don't need to worry about sequence.

### Deployment Flow

Terraform Phase:

terraform apply

↓

1. Authentication to NDO

↓

2. Read existing resources (data sources)

↓

3. Calculate dependency graph

↓

4. Create VRF-RCC-E in UpgradeTemplate1

↓

5. Create Contract in UpgradeTemplate1

↓

6. Create ANPs in each template

↓

7. Create BDs (parallel where possible)

↓

8. Create BD Subnets

↓

9. Create EPGs

↓

10. Create Site Associations

├── Site ANPs (Grafenwöhr and Kaiserslautern)

├── Site EPGs

└── Site BDs

↓

11. Create Physical Domain Associations

↓

12. Update Terraform state

↓

✅ RCC-E Infrastructure Ready

Python Phase:

python3 ipv6_binding_generator.py both

↓

1. Authenticate to NDO API

↓

2. Fetch Schema "AEDCE"

↓

3. Discover all AppProf-RCC-E EPGs

- Scan all 5 templates

- Find 33 IPv6 RCC-E EPGs

↓

4. Extract IPv4 Port Bindings

- Scan both site deployments

- Collect bindings from reference EPGs

- Filter out leaves 101/102

↓

5. Generate IPv6 Bindings

- Map IPv6 EPG → IPv4 reference

- Assign verified VLANs

- Filter by template/site (Grafenwöhr vs Kaiserslautern)

↓

6. Save to JSON file

- ipv6_rcc_e_port_bindings.json

↓

7. Deploy to NDO (if confirmed)

- Build EPG cache

- Create PATCH operations

- Deploy in batches of 50

- Progress tracking

↓

✅ Port Bindings Active

## API Interactions

### NDO REST API Endpoints

The Python script uses these API endpoints:

Authentication:

POST /api/v1/auth/login

Body: {"username": "admin", "password": "..."}

Response: {"token": "eyJ..."}

Schema Operations:

GET /api/v1/schemas

Response: {"schemas": [{"id": "...", "displayName": "AEDCE", ...}]}

GET /api/v1/schemas/{schema-id}

Response: Complete schema JSON with all templates, sites, BDs, EPGs

PATCH /api/v1/schemas/{schema-id}

Body: [{"op": "add", "path": "/sites/0/anps/0/epgs/0/staticPorts/-", "value": {...}}]

Sites Operations:

GET /api/v1/sites

Response: {"sites": [{"id": "...", "name": "AEDCG", ...}]}

### API Request Flow

1. Authentication Request

↓

2. Receive Token

↓

3. Add Token to Header: "Authorization: Bearer <token>"

↓

4. All subsequent requests use token

↓

5. GET requests: Read current state

↓

6. PATCH requests: Modify configuration

↓

7. Response: 200/202/204 = Success

### PATCH Operations for Port Bindings

Example PATCH operation:

{

"op": "add",

"path": "/sites/0/anps/0/epgs/0/staticPorts/-",

"value": {

"type": "vpc",

"path": "topology/pod-1/protpaths-111-112/pathep-[VPC_D1A-B]",

"portEncapVlan": 3021,

"deploymentImmediacy": "immediate",

"mode": "regular"

}

}

Path Components:

- /sites/0: First site in schema (Grafenwöhr)

- /anps/0: First ANP in site

- /epgs/0: First EPG in ANP

- /staticPorts/-: Append to staticPorts array

## Performance Considerations

### Terraform Performance

Creation Time per Resource Type:

| Resource Type | Time per Resource | Parallelism |
| --- | --- | --- |
| VRF | ~2 seconds | No |
| BD | ~3 seconds | Yes (up to 10) |
| BD Subnet | ~2 seconds | Yes |
| EPG | ~3 seconds | Yes |
| Site Association | ~2 seconds | Yes |
| Site BD | ~2 seconds | Yes |
| Domain Association | ~2 seconds | Yes |

Total for 308 resources: ~5-10 minutes (parallelism significantly helps)

Parallel Execution: Terraform creates up to 10 resources simultaneously where dependencies allow.

### Python Script Performance

Port Binding Deployment Statistics:

- Bindings per batch: 50

- Delay per batch: 0.5 seconds

- Total bindings: ~100

- Total batches: ~2

- Total deployment time: 2-3 minutes

Optimization Options:

- Increase batch size for faster deployment (risk: API timeout)

- Decrease batch size for more reliable deployment (slower)

- Adjust time.sleep() based on NDO performance

## Security Architecture

### Network Segmentation

Isolation Model:

- 33 separate security zones (EPGs)

- Default: No communication between zones

- Contract-based policy: Explicit allow rules required

- Micro-segmentation ready for RCC-E

Benefits for RCC-E:

- Contains security breaches within one service

- Limits lateral movement

- Compliance-friendly (each service isolated)

- Audit-ready (all communication explicit)

### Public-Facing Services

Three EPGs marked as PUBLIC (internet-accessible):

- EPG-RWEB-PROXY (d7) - Reverse Web Proxy

- EPG-FWEB-PROXY (d8) - Forward Web Proxy

- EPG-WEB-SVR (e4) - Web Servers

Required Security Measures:

- Additional firewall rules

- Enhanced monitoring and logging

- Intrusion detection/prevention

- Regular vulnerability scanning

- DDoS protection

Regional Cyber Center Europe (RCC-E)

Last Updated: November 17, 2025